TwoZ Setup directions

Ensure you have the right version of the cnc interface for your prepreprocessor.

Copy the twoz.pre file into your techno cnc interface folder.

By default, this folder is located at

C:\program files\techno cnc interface\

Once the file is copied, open the CNC interface

Left click and HOLD the preprocess button. This will bring up the pre-preprocess menu. The available prepreprocessors will be listed on the left side.

If you see no prepreprocessors then the file was not successfully copied to the proper directory.

Click in the left pane to select the prepreprocessor. Click the arrow in between the panes to move it to the right pane

Click in the right pane on the prepreprocessor. Click on settings and set the appropriate values for each setting.

Click Save, then close.

The prepreprocessor is now properly configured.

The cnc interface must now be set up properly.

Click on SETUP

Click on ADVANCED

Check Automatic toolchanger present

Check Ignore Toolchanger Drawbar Sensor

Click on the left on TOOLS

For Tool 1 AND 2 under the SPECIAL column, use the backspace key to remove the “0” and in its place, put the number 10

For tool 2 ONLY , under the X and Y offset, put the distance between the center of a tool in spindle 1 and 2. It is likely that you will have a negative value for X and if the Y value is not zero, if the A axis is closer to the front of the table than the Z axis, the Y value will be positive.

Click OK in the lower left. You will need to restart the software.

The configuration is now complete.

To run the system:

When the software starts up, identify the tool as tool 1 (Click TOOL, identify tool in chuck)

Properly zero the Z axis to X and Y zero with the Z zero on top of the material

Properly Zero the A axis on top of the material.

Preprocess the file

Run